Artificial Intelligent

LAB - 5

Robotics Scenarios and Simulations in Python

Scenario 1: Simple 2D Robot Motion

Problem

Simulate a robot that moves in a 2D plane in a straight line with constant speed and direction.

Robot's Solution

The robot uses simple kinematic equations to move based on its orientation (theta) and speed.

Code & Logic

x, y = 0, 0  Start position

theta = np.pi / 4  Angle: 45 degrees

v = 1  Constant speed

Time steps

for t in time:

x += v * np.cos(theta) * 0.1

y += v * np.sin(theta) * 0.1

- cos(theta) and sin(theta) determine direction.

- Position updated every small time increment (0.1).

- The path is plotted using matplotlib.

Code:

import numpy as np

import matplotlib.pyplot as plt

x, y = 0, 0

theta = np.pi / 4  # 45 degrees

v = 1  # velocity

time = np.linspace(0, 10, 100)

x_path, y_path = [], []

for t in time:

x += v * np.cos(theta) * 0.1

y += v * np.sin(theta) * 0.1

x_path.append(x)

y_path.append(y)

plt.plot(x_path, y_path, label='Robot Path')

plt.title('Simple 2D Robot Motion')

plt.xlabel('X')

plt.ylabel('Y')

plt.legend()

plt.grid(True)

plt.axis('equal')

plt.show()

Scenario 2: Obstacle Avoidance

Problem

A robot moves in a grid and must avoid static obstacles by changing direction.

Robot's Solution

If an obstacle is detected in the next cell, the robot turns 90 degrees clockwise.

Code & Logic

directions = [(1, 0), (0, 1), (-1, 0), (0, -1)]  # Right, Up, Left, Down

for _ in range(15):

next_x = x[-1] + directions[dir_index][0]

next_y = y[-1] + directions[dir_index][1]

if (next_x, next_y) in obstacles:

dir_index = (dir_index + 1) % 4  # Turn right

continue

- Maintains direction index.

- Checks next position for obstacles.

- If blocked, rotates right.

Code:

import matplotlib.pyplot as plt

x, y = [0], [0]

directions = [(1, 0), (0, 1), (-1, 0), (0, -1)]  # Right, Up, Left, Down

dir_index = 0

obstacles = {(3, 0), (4, 0), (0, 5)}

for _ in range(15):

next_x = x[-1] + directions[dir_index][0]

next_y = y[-1] + directions[dir_index][1]

if (next_x, next_y) in obstacles:

dir_index = (dir_index + 1) % 4  # Turn right

continue

x.append(next_x)

y.append(next_y)

for ox, oy in obstacles:

plt.plot(ox, oy, 'rs')  # obstacle

plt.plot(x, y, 'bo-')

plt.title('Obstacle Avoidance')

plt.grid(True)

plt.axis('equal')

plt.show()

Scenario 3: Line Following using PID Controller

Problem

The robot must follow a curved line path using sensor feedback.

Robot's Solution

A PID controller adjusts the robot’s movement to reduce error between desired and actual path.

Code & Logic

error = y_desired[i] - y_robot

y_robot += Kp * error + Ki * error_sum + Kd * derivative

- Kp, Ki, Kd: PID gains.

- error: difference between current position and target.

- PID uses error, its sum (I), and rate of change (D) to update the path.

Code:

import numpy as np

import matplotlib.pyplot as plt

# Desired path

x = np.linspace(0, 50, 500)

y_desired = 8 * np.sin(0.1 * x)

# PID Controller parameters

Kp = 0.5

Ki = 0.01

Kd = 0.1

y_robot = 0

y_path = []

error_sum = 0

prev_error = 0

for i in range(len(x)):

error = y_desired[i] - y_robot

error_sum += error

derivative = error - prev_error

y_robot += Kp * error + Ki * error_sum + Kd * derivative

prev_error = error

y_path.append(y_robot)

plt.plot(x, y_desired, 'r--', label="Desired Path")

plt.plot(x, y_path, 'b-', label="Robot Path")

plt.legend()

plt.title("Line Following Robot (PID)")

plt.grid(True)

plt.show()

Scenario 4: Path Planning with A*

Problem

Find the shortest path from start to goal in a grid with obstacles.

Robot's Solution

Uses A* search algorithm with a heuristic to find optimal path.

Code & Logic

heapq.heappush(open_list, (f, cost, current, path))

- f = cost + heuristic.

- Heuristic: Manhattan distance.

- Maintains open list as a priority queue using heapq.

Code:

import heapq

grid = [[0, 0, 0, 1, 0],

[1, 1, 0, 1, 0],

[0, 0, 0, 0, 0],

[0, 1, 1, 1, 1],

[0, 0, 0, 0, 0]]

start = (0, 0)

goal = (4, 4)

def heuristic(a, b):

return abs(a[0] - b[0]) + abs(a[1] - b[1])

def a_star(grid, start, goal):

open_list = []

heapq.heappush(open_list, (0 + heuristic(start, goal), 0, start, []))

visited = set()

while open_list:

_, cost, current, path = heapq.heappop(open_list)

if current in visited:

continue

visited.add(current)

path = path + [current]

if current == goal:

return path

for dx, dy in [(0,1), (1,0), (-1,0), (0,-1)]:

nx, ny = current[0] + dx, current[1] + dy

if 0 <= nx < len(grid) and 0 <= ny < len(grid[0]):

if grid[nx][ny] == 0:

heapq.heappush(open_list, (

cost + 1 + heuristic((nx, ny), goal),

cost + 1, (nx, ny), path))

return []

path = a_star(grid, start, goal)

for i in range(len(grid)):

for j in range(len(grid[0])):

if (i, j) == start:

print("S", end=" ")

elif (i, j) == goal:

print("G", end=" ")

elif (i, j) in path:

print("*", end=" ")

elif grid[i][j] == 1:

print("#", end=" ")

else:

print(".", end=" ")

print()

Scenario 5: Two Robots Avoiding Collision

Problem

Two robots moving toward each other in shared space must avoid collision.

Robot's Solution

Each robot monitors the other's position. If they get close, both stop.

Code & Logic

if abs(r1[0] - r2[0]) <= 1 and abs(r1[1] - r2[1]) <= 1:

dx1 = dy1 = dx2 = dy2 = 0

- Both robots use simple logic to approach each other.

- If their positions are close, movement stops.

Code:

import matplotlib.pyplot as plt

robot1 = [[0, 0]]

robot2 = [[10, 10]]

for _ in range(20):

r1 = robot1[-1]

r2 = robot2[-1]

# Simple rule: move towards each other

dx1 = 1 if r1[0] < r2[0] else -1

dy1 = 1 if r1[1] < r2[1] else -1

dx2 = -dx1

dy2 = -dy1

# Collision avoidance

if abs(r1[0] - r2[0]) <= 1 and abs(r1[1] - r2[1]) <= 1:

dx1 = dy1 = dx2 = dy2 = 0

robot1.append([r1[0] + dx1, r1[1] + dy1])

robot2.append([r2[0] + dx2, r2[1] + dy2])

robot1 = list(zip(*robot1))

robot2 = list(zip(*robot2))

plt.plot(*robot1, 'r-o', label='Robot 1')

plt.plot(*robot2, 'b-o', label='Robot 2')

plt.title('Two Robots Avoiding Collision')

plt.legend()

plt.grid(True)

plt.axis('equal')

plt.show()

Scenario 6: 2-Link Planar Arm

Problem

Simulate forward kinematics of a 2-link robot arm.

Robot's Solution

Uses trigonometric equations to compute end-effector position.

Code & Logic

x_elbow = l1 * np.cos(t1)

x_tip = x_elbow + l2 * np.cos(t1 + t2)

- Calculates elbow and tip position.

- Plots the trajectory of the tip (end-effector path).

Code:

import numpy as np

import matplotlib.pyplot as plt

l1, l2 = 5, 3

theta1_range = np.linspace(0, np.pi, 100)

theta2_range = np.linspace(0, np.pi/2, 100)

x1, y1 = [], []

x2, y2 = [], []

for t1, t2 in zip(theta1_range, theta2_range):

x_elbow = l1 * np.cos(t1)

y_elbow = l1 * np.sin(t1)

x_tip = x_elbow + l2 * np.cos(t1 + t2)

y_tip = y_elbow + l2 * np.sin(t1 + t2)

x1.append(x_elbow)

y1.append(y_elbow)

x2.append(x_tip)

y2.append(y_tip)

plt.plot(x2, y2, 'b-', label='End Effector Path')

plt.title('2-Link Robot Arm Motion')

plt.xlabel('X')

plt.ylabel('Y')

plt.legend()

plt.grid(True)

plt.axis('equal')

plt.show()

🔹 Scenario 9: SLAM Grid Mapping (Static)

✅ Problem

Simulate robot mapping its environment as it moves.

✅ Robot's Solution

Robot uses simulated scanning in four directions and updates occupancy grid.

✅ Code & Logic

if grid[scan_x][scan_y] == 1:

grid[scan_x][scan_y] = 2  # mark as seen obstacle

- grid=1: obstacle

- grid=2: observed obstacle by sensor

- Robot moves and performs 4-direction scan.

🔹 Scenario 10: Wall Avoidance via Reinforcement Learning (Q-Learning)

✅ Problem

Robot learns to avoid hitting walls using Q-learning.

✅ Robot's Solution

Learns action values (Q-values) based on rewards and updates its knowledge over episodes.

✅ Code & Logic

q_table[pos[0], pos[1], action] += alpha * (reward + gamma * best_next - q_table[pos[0], pos[1], action])

- Actions: up, down, left, right.

- Q-table stores expected rewards.

- Robot gets negative rewards for walls and learns best routes.

Would you like me to format all these into a PDF Lab Manual or package them as a Python project folder with .py files and documentation?